The information in this section is an extract from the DOORS module identified by the following information.

DOORS module path: /TERMA DOORS/IDS/Naval/Naval C2/Common Features/2. System Level Requirements/

DOORS module name: 2274145-DT

DOORS module description: Naval Sensors System Requirements

DOORS baseline version: 1.4

DOORS view: RDN view

Contents

1	Naval Sensors	4

1.1	Introduction	4

1.1.1	Purpose	4

1.1.2	Application	4

1.2	References	4

1.3	Definitions	4

1.4	Reader's guide	5

1.4.1	Form	5

1.4.2	Product Tiers	5

1.5	Roles/Personas	5

1.6	Control and Monitor Sub-systems	6

1.6.1	Control and Monitor Surveillance Radar	6

1.6.2	Control and Monitor IFF Interrogator	7

1.6.3	Control and Monitor ESM System	8

1.7	System Interfaces	8

1.7.1	Target Sensors	8

# Naval Sensors

## Introduction

### Purpose

Describes functionality related to Control of target sensors in Naval C2 systems via User Scenarios.

### Application

Used for communicating needs to system engineers, software developers, testers, and managers for Naval C2 products and projects.

## References

Ref.	Doc. No.	Title

[BNC]	2017969-DT	Basic Naval C2 System Requirements

[NSES]	2274145-DR	Naval Sensors Stakeholder Requirements

[MSSR]	30016390	MSSR 2000 I, IDD External Interf. MSSR 2000 I (Airbus/Casidian)

30005622	MSSR 2000 I, Interface Control Document – ASTERIX Implementation (Airbus)

[PHOBOS]	QR020-M4	Terma Interface Control Tactical Threat System

[SCANTER]	264826-DI	SCANTER 4103 Transceiver Control Protocol Data Definition

386300-RI	SCANTER 5000/6000 Series Interface Overview

386300-DI	SCANTER 5000/6000 Series Transceiver Interface Specification

386308-DI	SCANTER 6000 Series Transceiver Control Protocol Data Definition

304124-SI	SCANTER Network Video Protocol

502074-DI	SCANTER Radar System Protocol

303949-SI	SCANTER Track Management Protocol

304284-SI	SCANTER Plot Management Protocol

## Definitions

Term	Definition

AAZ	Automatic Acquisition Zone

ADT	Automatic Detection & Tracking

BITE	Built-In Test Equipment

ESM	Electronic Support Measures

EW	Electronic Warfare

IFF	Identification Friend or Foe

LAN	Local Area Network

MIP	Mode Interlace Patterns

MSSR	Monopulse Secondary Surveillance Radar

MTI	Moving Target Indicator

NAAZ	Non-Automatic Acquisition Zone

NTZ	Non-Tracking Zone

RF	Radio Frequency

SE	System Engineer

STC	Sensitivity Time Control

SW	Software

## Reader's guide

### Form

This document must be read in conjunction with the Basic Naval C2 User Scenarios [BNC]. The User Scenarios in this document is intended to augment and be placed in the same hierarchical structure as defined in [BNC].

Requirements are given as user scenarios, and only aspects poorly suited for user scenarios are given as line requirements.

The terms "shall" and "must" are used to specify mandatory aspects (i.e. for essential features), whereas the term "should", is used to denote desirable but non-essential aspects. The term "shall not" and "must not" are used to denote aspects that are unacceptable or prohibited.

For the user scenarios, the form is a base statement:

As a <role>, I want <some goal>, so that <some reason/business value>

followed by some comments giving context to the base statement.

This shall be augmented with accept criteria that defines what must be fulfilled for the delivered functionality to be accepted. Accept criteria pertains to the solution, not the process.

To get sufficient coverage of needs, user scenarios can be at multiple abstraction levels. Each subsection shall act as an Accept criteria for the overlying user scenario. Only the lowest level of Accept criteria are marked as Requirement.

Please note that a user scenario is not meant to be a complete specification. Rather, it shall be a vehicle for conversation between developers/architects and "customers", and the accept criteria shall specify only the truly important parts - not everything the author can possibly think of.

### Product Tiers

When this document is viewed with product tier markers, the following definition applies:

Code	Definition

D	Part of the RDN platforms

P	Part of the Patrol product tier

L	Part of the Light Combat product tier

## Roles/Personas

This section outlines the user types that are stakeholders in a Naval C2 system.

Role	Definition

Operator	Normal non-privileged runtime user of the system.

EW Operator	Operator in charge of handling EW lines

System Configurator	Supplier employee that (at pre-delivery time or at least off-line) configures or sets up the system. Normally SW developers or SEs

## Control and Monitor Sub-systems

[NSES167] As an Operator, I want to control and monitor the sub-systems available in my configuration.

### Control and Monitor Surveillance Radar

[NSES169] As an Operator, I want to control and monitor the Radar sub-systems available in my configuration.

Configuration Variance

NSE381	•	The availability of Radar sub-systems shall be configurable.

#### SCANTER Radar

##### Control and Monitor SCANTER Radar

[NSES181] As an Operator, I want to control and monitor the SCANTER Radar to suit my mission goal.

Accept Criteria

NSE501	•	I want to be able to turn the radar power on and off (not supported for SCANTER 2602).

NSE502	•	I want to be able to start and stop the radar rotation.

NSE503	•	I want to be able to switch transmission on and off, independent of the rotation.

NSE504	•	I want to be able to select a predefined radar profile, giving the range, mission focus and weather focus I need, as I may not have the necessary training or experience to do it myself.

NSE505	•	I want to be able to enable and disable distribution of plots received from the SCANTER Radar. Distribution of plots received from the SCANTER Radar shall automatically be disabled when the radar profile is changed.

NSE506	•	I want to be able to view the status of the radar.

NSE508	•	I want to be able to view BITE results.

NSE509	•	If connected, I want to be able to view the status of the SCANTER, e.g. rotation speed and error status, but not necessarily very low-level BITE errors.

NSE510	•	I want to be able to define north-referenced or heading referenced silence sectors (sectors where transmission is off).

NSE511	•	I want to be able to control limit transmissions in north referenced or heading referenced directions which create false echoes/radhaz.

NSE512	•	I want to be able to define up to 16 transmission sectors with coverage, mode and power-reduction parameters for the SCANTER Radar.

NSE513	•	I want to be able to control and reset the surface radar video gain, STC and clutter control for the SCANTER Radar.

##### Control the SCANTER Radar Tracker

[NSES182] As an Operator, I want to control the SCANTER Radar tracker to optimize its tracking capabilities to my mission.

Accept Criteria

NSE524	•	I want to be able to define geographically fixed polygon-type Non-Tracking Zones for Normal mode and for MTI mode independently for the SCANTER Radar.

NSE525	•	I want to be able to define geographically fixed polygon-type Non-Automatic Acquisition Zones for Normal mode and for MTI mode independently for the SCANTER Radar.

##### Target Classification from SCANTER

[NSES219] As an Operator, I want to get target classification from the SCANTER, so I better understand what the Radar is tracking.

Accept Criteria

NSE761	•	I want to be able to monitor target classification probabilities for SCANTER tracks.

### Control and Monitor IFF Interrogator

[NSES184] As an Operator, I want to use an IFF interrogator, so it can help me identify entities.

Configuration Variance

NSE531	•	The availability of an IFF Interrogator shall be configurable.

#### Control and Monitor MSSR2000I IFF Interrogator

[NSES185] As an Operator, I want to control the MSSR2000I IFF Interrogator, so I can interrogate and receive information.

Accept Criteria

NSE537	•	As an operator I want to be able to turn IFF interrogator transmission on/off.

NSE539	•	On the MSSR2000 IFF, I want to be able to select 1 to 8 MIPs (Mode Interlace Patterns) to be executed in turn, with one antenna rotation each. A MIP is a combination of IFF modes, normally up to 3.

NSE540	•	As a configurator, I want to be able to set descriptive presentation names for each MIP. For example, that MIP 0 is "M1 + M2", MIP 1 is "M1 + M2 + MS", etc. The mapping is project/ship specific.

NSE541	•	I want to be able to import descriptive presentation names for each MIP.

NSE543	•	Replies in at least the following IFF modes shall be supported: 1, 2, 3/A, C, S.

NSE544	•	Replies in at least the following encrypted military IFF modes should be supported: 5.

NSE545	•	I want to be able to see the status of the IFF interrogator.

NSE767	•	I want to be able to see the health status indicator of the radar, transmission and system time.

NSE550	•	I want to be able to define north-referenced silence sectors and switch them on and off.

### Control and Monitor ESM System

[NSES188] As an EW Operator, I want to control and monitor an ESM system, so I can receive passive tracking information.

Configuration Variance

NSE565	•	The availability of an ESM system shall be configurable.

#### Control and Monitor a Phobos ESM system

[NSES189] As an EW Operator, I want to control and monitor the Phobos ESM system, so I can evaluate if it is configured correctly and working.

----------------------------------------

All emitters forwarded from the ESM system are automatically created as internal bearing lines.

Setting type to Unknown is appropriate because the Type field from Phobos does not have an obvious mapping.

Accept Criteria

NSE572	•	I want to be able to see Phobos link status.

NSE573	•	I want to be able to see the number of EW bearing lines reported by the Phobos system.

NSE574	•	I want to be able to see the library used by the Phobos system.

NSE575	•	I want the track management type of EW bearing lines received from Phobos ESM to be set to Unknown.

## System Interfaces

[NSES199] As a System Configurator, I want that the system implements specified subsystem interfaces, so I can ensure proper integration of all interfaced subsystems.

### Target Sensors

#### SCANTER

[NSES203] As a System Configurator, I want that the system implements the specified SCANTER interface, so I can ensure proper integration with the subsystem.

Configuration Variance

NSE664	•	By configuration the system shall be able to support SCANTER 2602.

NSE666	•	By configuration the system shall be able to support SCANTER 4603.

NSE667	•	By configuration the system shall be able to support SCANTER 6002.

NSE669	•	I want the system to implement the SCANTER 6002 Helicopter Approach and Surveillance Radar System interface in compliance with Document Number 386300-RI, 386300-DI, 386308-DI, 386304-006-DI, 304124-SI, 502074-DI, 303949-SI, 1026364-DI, 1024613-DI, and 304284-SI.

#### IFF Interrogator (MSSR 2000 I)

[NSES207] As a System Configurator, I want that the system implements the specified MSSR 2000 I interface, so I can ensure proper integration with the subsystem.

Accept Criteria

NSE698	•	I want the system to implement the MSSR 2000 I interface in compliance with 30016390 and 30005622.

NSE765	•	I want the system to be able to read ADS-B data combined with IFF on the same port.

#### ESM System (Phobos)

[NSES209] As a System Configurator, I want that the system implements the specified Phobos ESM System interface, so I can ensure proper integration with the subsystem.

Accept Criteria

NSE710	•	I want the system to implement the Phobos ESM System interface in compliance with ICD Terma Interface Control Tactical Threat System QR020-M4.